Printing Instructions Update

- Finish Drafting and Change Workspace to Layout 1

When working in spatial.NET, you will be drafting and modifying objects in MODEL space.  When you are ready to print, please refresh the workspace. The area you want to print will be shown in the current MODEL working space.

When ready to print, right click on the Layout 1 tab the bottom of the screen. A view of the screen extents will appear.

- Setup the Print Options (PAPER space)

When the workspace changes to Layout1, the paper is shown in the PAPER space.

First we will set up the paper size in the PAPER space.

Right click on the Layout tab and choose Page Setup Manager.

Select Layout 1 and click Modify.

Choose the paper size needed and the printer. Alter other settings as needed. Click OK when finished.

EX:

Printer: DWG to PDF.pc3;   Paper Size: ANSI A (11.00*8.50 inches);

Plot Area: Layout;   Plot Scale: 1:1.  These two values should be used as default setting.

As shown in the following picture, viewport contains the models you draft. The size of viewport could only be changed in PAPER space. The models shown in viewport could only be changed in MODEL space.

Paper Area is the paper boundary according to the paper size you choose.

- Add title block in PAPER space

Use Insert…Block to add a title block to your drawing.

Click Browse and select the path of the Block file. Choose the plot border according to the paper size you set (EX: ANSI A 11.0*8.5).  Path:

T:\ITS\Inventory\SPATIALinfo\SPATIALnet\templates\plot border

Click OK. Choose the insert point. The insert point locates at the NorthWestern Corner of the title Block. ZOOM IN and insert at the NorthWestern Corner of the PAPER AREA.

When you insert the block, you may be asked to set the attribute values ((1) Project No.  (2) HWY Name  (3) County Name  (4) SHEET Number  (5) Plot by (author)     (6) Plot Scale)). You can set them when you insert the block with command lines right now. Or you can just press " ENTER" to skip this process. You can set the values later in PAPER SPACE later.

The view port may not fit the title block. Click the viewport outline on the page and drag the blue grip to fit the screen after you insert the print border.

Set the title block. The file name and plot date will be generated. Therefore, you don't need to set these two values. They will be generated automatically while printing.

What you need to set is (1) Project No.  (2) HWY Name  (3) County Name  (4) SHEET Number  (5) Plot by (author)     (6) Plot Scale

In PAPER Space, click the title block. Then double click the attribute you want to edit. Input the value and press "OK".

- Locate Print Area (MODEL space)

After setting the title block, switch from PAPER space to MODEL space in the lower right corner of the screen.

Use the spatialNET Find options to locate what you wish to print, such as a boundary. Display the map view or use zoom to focus on the area.

Zoom in the area that you wish to see using the mouse.

If there is content on the map you wish to clear out, type the command layoff  (layer off) in the command window.

You will be prompted to select items on the layer you wish to turn off. After the map has been cleared of extra objects, press Enter to exit the layoff command.

Active the Viewport ToolBar in AutoCAD Tool Bar (Right Click on AutoCAD tool bar and select Viewport). You will see the Viewport tool bar as following. The number 0.0073 shows the curret plot scale in MODEL Space. Type in the scale you want. (Ex: Input 0.01, the plot scale will change to 1:100. Input 0.02, the scale will be 1:50)

Use File…Save As to save a copy of this as a dwg file for later reference. Or use File…Plot to send the view to a printer. ( Ctrl + P)